# Phone Box — Next Steps

Getting the companion app onto your iPhone, and connecting it to the box.

Last updated: 2026-07-24

# Where things stand

DONE — Box firmware updated with Bluetooth; the Bluetooth support library is installed on the box.

DONE — Battery-meter chip and SD-card features temporarily shelved (you do not have that hardware yet); the box runs fine without them.

DONE — Box is powered on and broadcasting as "PhoneBox" (confirmed in the LightBlue scanner app).

DONE — Phone app draft cleaned up on the PC; automatic checks pass (types + tests).

TO DO — Build the app on a Mac and install it on your iPhone (this document).

# Optional: prove the box exposes data (2 minutes, in LightBlue)

Not required, but it confirms the box is fully working before you spend time on the Mac build.

In LightBlue, tap "PhoneBox" to connect.

Look for a service whose ID starts with 6b9a7e00-…0001, containing characteristics ending 0002 through 0007.

Tap the one ending 0002 ("status") and tap Read. You should see a short piece of text like: {"st":"idle","rem":0,...}

If you see that text, the box is good. If connecting fails or the service is empty, stop and flag it.

# Step 1 — Get the project onto the Mac

The project lives in OneDrive, so the easiest route is to let OneDrive copy it for you.

## Option A — OneDrive (simplest)

On the Mac, install OneDrive from the Mac App Store and sign in with the SAME account as the PC (mah.e@northeastern.edu).

Wait for it to finish syncing. The whole "Phone Box" folder will appear on the Mac.

## Option B — USB stick

Copy the "Phone Box" folder to a USB drive on the PC.

Plug it into the Mac and copy the folder to the Desktop.

Note: Inside the app folder is a large sub-folder called "node_modules" (downloaded helper code). Do not rely on the PC copy of it — it has Windows-specific parts. You will rebuild it fresh on the Mac in Step 3. You can skip copying it to save time.

# Step 2 — One-time Mac setup (do once)

Install Xcode from the Mac App Store (Apple’s app-building tool; it is a large download).

Open Xcode, then Xcode menu → Settings → Accounts → "+" and sign in with your Apple ID. A free Apple ID is fine.

On the iPhone: Settings → Privacy & Security → Developer Mode → turn On (the phone restarts).

Connect the iPhone to the Mac with a cable, and tap "Trust" on the phone when prompted.

# Step 3 — Build the app and install it on the iPhone

Open the Terminal app on the Mac and run these one at a time. Replace the path in the first line with where the folder actually is (tip: type "cd " then drag the "app" folder onto the Terminal window to fill in the path).

cd "/path/to/Phone Box/app"

npm install                 # rebuilds the helper code, fresh for the Mac

npx expo prebuild --clean   # sets up the native iPhone project

npx expo run:ios --device   # builds it and installs onto your iPhone

The last command asks which device — pick your iPhone.

The first build is slow (several minutes). That is normal.

# Step 4 — If the build stops on a "signing" error

Apple requires every app to be "signed" with your identity. If the last command complains about signing:

Open the file the build created: app/ios/ and double-click the ".xcworkspace" file (opens in Xcode).

Click the project name at the top of the left sidebar.

Go to the "Signing & Capabilities" tab.

Under "Team", choose your name/Apple ID.

If it complains the name "com.phonebox.app" is taken, change it slightly (e.g. com.YOURNAME.phonebox), then run "npx expo prebuild --clean" and the run command again.

# Step 5 — Trust the app on the iPhone (free Apple ID only)

On the iPhone: Settings → General → VPN & Device Management.

Tap your developer profile, then tap Trust.

The app will now open.

# Step 6 — See it all work together

Power on the box (it broadcasts as "PhoneBox").

Open the app on the iPhone.

Allow Bluetooth when the phone asks.

Tap Connect — the app should find the box and show its live status.

# Good to know

Bluetooth does NOT work on the iPhone Simulator — you must use a real iPhone (that is why we use "--device").

This app has never been built before, so the first build may hit one or two errors — that is expected for a first run.

If a command prints red error text, copy that text and send it over; that is enough to pinpoint the fix.

The battery view will show "N/A" and the stats view "NO SD" until that hardware is added — this is intentional, not a bug.