# Phone Box — Feature & Ideation Suggestions

Date: 2026-07-20

Prepared for: Phone Box (CircuitPython touchscreen phone-lock box)

Purpose: Single consolidated reference for every feature idea and ideation output from

the working session — the premium-feature analysis plus the two deep-dives (usage tracking,

and greenlisting notifications). Focus: which additions would justify a higher retail price,

grounded in what the hardware actually supports. *This document supersedes the earlier

standalone deep-dive notes; the full content of both is folded into Deep-dive A and B below.*

## Executive summary

Phone Box is already more capable than most competitors — it has a touchscreen, a

servo latch, an emergency override, and a rechargeable battery with a level display, which

cheap boxes ($20–35) lack. In this category the premium gap is software, not hardware:

the products that sell for $59+ (Brick, kSafe, Unpluq) win on app connectivity, scheduling,

analytics, and accountability.

The pivotal fact: the **ESP32-S3's Wi-Fi + BLE radio is already in the bill of materials but

completely unused** (the firmware contains zero networking code). So the highest-value

premium features add software cost only, no BOM increase — the highest-margin way to

move upmarket.

Target: a credible move from the "dumb box" band (~$20–35) into the connected/branded

band ($59–89 one-time), with an optional family/accountability subscription

(~$60–100/yr) as the only market-validated recurring-revenue path.

## Market context (competitor research, 2026)

Price bands observed:

| Segment | Example | Price |
| --- | --- | --- |
| Budget "dumb" electronic boxes | ySky, generic Amazon | $20–35 |
| Mid-tier hardware upsell (charge-through) | Habit Control | ~$34 |
| Branded / connected premium | Brick ($59), kSafe | $59–79 |
| Self-contained ceiling | kSafe XL, Unpluq bundle | ~$79 |
| Family/accountability subscription | Unpluq Family | $99.95/yr |

What commands a premium (ranked, evidence-backed): (1) app connectivity with modes &

scheduling, (2) usage analytics / dashboards, (3) recurring/scheduled locks (start and

end times), (4) accountability / family / multi-device management, (5) charge-through (lock

while charging), (6) tamper resistance / "no override" integrity, (7) build quality /

design-as-object.

Willingness-to-pay signals: buyers explicitly pay for physical friction — app-only

blockers fail because they're "one tap" to bypass, which validates a hardware box. $59 is

framed as a low-risk "experiment" for people who already failed with free Screen Time.

Complaints about cheap boxes: flimsy build, weak tamper resistance, easily defeated, no

scheduling.

Recurring revenue: a one-time premium price is the safe default (Brick markets "no

subscription, ever" as a feature); a family/accountability tier is the only validated

recurring model in the category (Unpluq).

## Unused hardware = premium runway

| Asset | State | Enables |
| --- | --- | --- |
| Wi-Fi + BLE radio (ESP32-S3) | In BOM, zero code uses it (grep-verified) | Companion app, scheduling, accountability, OTA — no BOM cost |
| microSD / TF slot (4-bit SDIO, GPIO13–18) | Present, unused | Local session history for analytics |
| Free I²C bus @ 0x40 | Unused | INA219/INA226 → real current/power metering |
| Wall-clock time (RTC / NTP) | None (countdown only) | Scheduled / recurring locks; dated history |

All feature ideas below build on the existing modular architecture: config-as-single-source

(lock_config.py), per-peripheral driver modules (lock_servo.py, lock_battery.py, …),

an explicit state machine (lock_controller.py), view/scene rendering (lock_ui.py), and

versioned NVM persistence (lock_settings.py). New peripherals slot in as sibling modules

and new screens — not rewrites.

## Feature suggestions, ranked

### Tier 1 — Software only, no BOM increase (highest margin)

Companion app over BLE/Wi-Fi. *Builds on:* the unused ESP32-S3 radio; a new sibling

driver module + a new view in lock_ui.py + a pairing screen. *Why premium:* this is the

entire $59 Brick / Unpluq value prop; it unlocks items 2, 3, 4, 6 and the notification

greenlist feature. *Effort:* high. **This is the keystone — most other premium features

depend on it.**

Scheduled / recurring locks. *Builds on:* the state machine + lock_settings.py;

needs wall-clock time (ESP32-S3 internal RTC + NTP once Wi-Fi exists). *Why premium:* a

named premium feature vs. a single manual countdown ("lock every night 10pm–7am").

*Effort:* medium.

Usage analytics / focus streaks. *Builds on:* the state transitions in

lock_controller.py as the event source; store to SD or flash; a new stats view. *Why

premium:* Brick markets its dashboard as a differentiator. *Effort:* medium.

Accountability / partner / family mode. *Builds on:* the existing override logic +

connectivity (item 1). *Why premium:* the clearest willingness-to-pay-more lever and the

only validated subscription path (Unpluq Family ~$100/yr). *Effort:* high.

Multiple lock profiles / modes. *Builds on:* the versioned NVM layout in

lock_settings.py (magic-byte bump) + the existing settings-detail UI. *Why premium:*

household / multi-use-case value. *Effort:* low.

OTA firmware updates. *Builds on:* connectivity (item 1). *Why premium:* post-purchase

feature delivery → recurring-value / subscription story. *Effort:* high.

### Tier 2 — Low-cost hardware adds ($1–3 BOM) with a quality/security story

Charge-through (lock the phone while it charges). *Builds on:* the enclosure + a

pass-through USB port; no firmware dependency. *Why premium:* the exact $20→$34 upsell in

the market (Habit Control). *BOM:* ~$1–3.

Real current/power metering. *Builds on:* the free I²C bus @0x40 for an INA219/INA226;

mirrors the lock_battery.py module pattern; replaces today's coarse watt estimate with

true runtime-remaining. *BOM:* ~$1–3.

Tamper / lid-open detection + alarm. *Builds on:* the existing sense-button pattern

(BTN_LOCK_PIN); add a reed/hall switch + buzzer on a free GPIO. *Why premium:* directly

answers the "easily defeated" complaint that penalizes cheap boxes. *BOM:* ~$1.

## Deep-dive A — Usage tracking: app vs. SD card

Question: how to log focus sessions (start, duration, completed vs. overridden, streaks)?

TL;DR: build SD logging first, then add a BLE app that reads from it. They are

complementary, not either/or. SD is the cheapest, most reliable way to *capture* data; the

app is how you *present and monetize* it. The event source for both already exists — the

state transitions in lock_controller.py (idle → running → done, plus override events).

Build the "emit a session record" seam once; SD and app are just two sinks for it.

The data to capture (same for both methods). Per session: set_duration,

actual_locked_time, completed vs overridden, override_press_count, and (if a time

source exists) start_timestamp. Small: ~30–60 bytes/session.

### Method A — SD card (local logging)

*How it works.* On each done/override transition, append a CSV/JSON-lines row to a file on

a microSD card. Read it later by removing the card or over USB.

*Hardware reality (verified).*

The Waveshare ESP32-S3-Touch-LCD-1.47 has an onboard TF/microSD slot, wired as a **4-bit

SDIO (SD/MMC) interface on GPIO13–18** (CLK, CMD, D0–D3) — consistent with the 13-18 SD

note in lock_config.py. The ESP32-S3 has a native SD/MMC host peripheral, so 4-bit SDIO

is supported.

CircuitPython path: sdioio for 4-bit SDIO (fastest), or sdcardio for SPI mode. Confirm

they're compiled into this board's CP 10.2.1 build before relying on them.

Cost: the slot is already on the board (no BOM change); a microSD card is ~$3–5.

*Key advantage — no CIRCUITPY conflict.* An external SD card mounted from code.py is

independent of the internal CIRCUITPY filesystem, so firmware can write to it freely at any

time, USB-connected or not. It does not hit the storage.remount / USB-vs-code write

conflict, and it does not risk the CIRCUITPY FAT corruption / read-only lockups that

already recur on this board.

*Weaknesses.* Presentation is manual (read a CSV over USB, or build an on-device stats view);

no absolute time without a clock (the ESP32-S3 internal RTC resets on power loss with no

backup cell, so logs are relative durations unless a ~$1–2 I²C RTC is added or time comes

from NTP/app); a card can be removed/lost, and write-during-power-loss can corrupt the last

row (mitigate with flush-per-record + append-only). *Effort:* low–medium, firmware only.

### Method B — Companion app (uses the currently-unused radio)

Three shapes, cheapest → richest:

B1 — BLE, phone app, no internet. Box is a BLE peripheral exposing a GATT service;

the phone app reads/subscribes to session records. CircuitPython has native _bleio on

ESP32-S3 (full peripheral support, CP 9.1+; this box runs CP 10.2.1). *Pros:* offline,

private, no backend/hosting; phone supplies real timestamps; rich UI. *Cons:* must build

iOS and Android apps; BLE pairing/reconnect UX is fiddly.

B2 — Wi-Fi, local (no cloud). Box hosts a tiny web server; phone browser views stats

on the same network. *Pros:* no native app; NTP gives real time. *Cons:* user must join

the box's AP / shared Wi-Fi; clunky; not a "product" feel.

B3 — Wi-Fi + cloud backend. Box pushes records to a server; web/app dashboards,

multi-device, remote accountability/family. *Pros:* richest; the only path to the validated

recurring-revenue tier. *Cons:* highest cost/liability — backend, accounts, security,

privacy, ongoing hosting cost.

*Cost:* no BOM change (radio present); real cost is software + maintenance. *Effort:* medium

(B1) to high (B3).

### Side-by-side

| Dimension | SD card | BLE app (B1) | Wi-Fi cloud (B3) |
| --- | --- | --- | --- |
| BOM cost | ~$3–5 card (slot exists) | $0 (radio exists) | $0 hardware |
| Ongoing cost | none | none | backend hosting, forever |
| Eng. effort | low (firmware only) | medium (2 apps + BLE) | high (apps + backend + accounts) |
| Data presentation | manual CSV / on-device screen | rich, on phone | rich, anywhere |
| Works offline | yes | yes (local) | no |
| Privacy | fully local | local | data leaves device |
| Real timestamps | needs RTC (~$1–2) or app | from phone | from NTP |
| Reliability | robust; no CIRCUITPY/FAT conflict | pairing drops | network + backend outages |
| Premium / monetization fit | weak alone | strong (Brick-tier) | strongest (subscription) |
| Depends on other features | none | connectivity work | connectivity + infra |

### Recommendation & sequencing

Now — SD logging. Add a session-record emit at the lock_controller.py transitions

and an sdioio/sdcardio sink. Ships real habit data for ~$3, firmware only, no

CIRCUITPY-corruption risk. Add a simple on-device stats view (lock_ui.py).

Next — BLE app (B1). Read the same records over BLE into a phone app for the

dashboards/streaks that justify the $59–89 band. SD stays the local source-of-truth; the

app is a viewer/sync layer, which de-risks the app work.

Later, only for recurring revenue — Wi-Fi/cloud (B3). For remote accountability/

family, accepting the permanent backend cost and privacy burden.

Timestamps: add a ~$1–2 I²C RTC if dated history matters before the app exists;

otherwise get time from the phone (B1) or NTP (B2/B3).

*Open items:* confirm this board's CP 10.2.1 build includes sdioio/sdcardio and _bleio;

confirm the TF slot's exact pin mode (SDIO 4-bit vs SPI) against the schematic; verify on

hardware. No firmware was changed for this analysis.

## Deep-dive B — Greenlisting notifications to unlock the box

Question: could certain whitelisted ("greenlisted") notifications — e.g. a call from a

specific person, or a chosen app — release the box early?

Short answer: yes on Android (fully), partial on iOS. It's an extension of the

companion-app / connectivity track, not standalone. The box side is trivial; the entire

difficulty is phone-platform notification access, which differs sharply by OS.

Why it's possible at all. The phone is sealed in the box, but plastic/wood doesn't block

RF — the phone keeps its cellular/Wi-Fi to the internet and its BLE link to the box. So a

companion app on the phone can keep running while locked inside and signal the box. Flow:

greenlisted notification arrives → app detects it → app signals the box over BLE → box either

(a) displays the alert on its screen / buzzes, or (b) releases the latch.

Box side — already built. Early release already exists: release_lock() in

lock_servo.py and the go_done transition in lock_controller.py. A BLE "unlock" command

just calls the existing path — no new mechanism. The box's touchscreen (lock_ui.py) makes

an "alert-through" overlay (show *"Call from Mom"* without unlocking) a natural addition.

Phone side — the real constraint.

Android — feasible. NotificationListenerService (user grants "notification access")

reads all notifications and filters by app package and/or sender, then signals the box.

Supports greenlisting specific apps *or* contacts.

iOS — restricted by design. Third-party apps cannot read other apps' notifications.

Practical options, weakest constraint first: incoming calls from a greenlisted contact

via CallKit/VoIP; the app's own push notifications only; **iOS 26.3 (2026) notification

forwarding to third-party accessories** — a real path where the box could receive forwarded

notifications, but EU-only (Digital Markets Act) and bound by strict use rules; or a

relay workaround routing a chosen contact's calls/SMS through a service (e.g. Twilio)

the app/box watches (clunky; adds infra). Net: on iOS, reliably do "greenlist a contact's

calls," with full notification greenlisting only for EU users on iOS 26.3+. Be honest about

this gap in marketing.

Design recommendation.

Two response modes, chosen per greenlist entry: *alert-through* (default — box stays

locked, shows the alert on screen / buzzes; keeps the focus contract intact) and

*auto-unlock* (releases the latch via release_lock()).

Greenlist config lives in the app (by app and/or contact).

Android-first; ship iOS as calls-via-CallKit + EU forwarding, clearly scoped.

Integrity caveats — this is a *new early-unlock path*: rate-limit it (a chatty app

shouldn't pop the box open); it's also a self-deception loophole (whitelisting a junk app

to cheat focus). Keep the existing press-count override as the true emergency path and

treat greenlist as a convenience layer, defaulting to alert-through.

*Dependencies & effort:* depends on the companion-app/BLE feature (items 1/4). Effort medium

on Android (notification listener + BLE command + box handler + alert overlay); high for full

iOS parity.

## Recommended sequencing (roadmap)

SD logging (Deep-dive A) — cheap, low-risk, ships real habit data; adds an on-device

stats view. ~$3 card, firmware only.

Companion app over BLE (item 1) — the keystone; turns the SD data into

dashboards/streaks and enables everything below.

Scheduled/recurring locks (item 2) + profiles (item 5) — high value, moderate

effort once connectivity + time exist.

Accountability / family mode (item 4) — turns on the validated subscription tier.

Greenlist-to-unlock (Deep-dive B) — layered on the app; alert-through first.

OTA (item 6) — supports ongoing feature delivery.

Tier-2 hardware adds (items 7–9) — charge-through, current sensor, tamper/alarm —

folded into the next enclosure revision, independent of the software track.

Strategic caveat: the connectivity track re-introduces the radio that earlier cost-down

work aimed to remove. This is a deliberate up-market bet (higher price, higher margin via

software), the opposite direction from BOM reduction. The Tier-2 hardware adds are far

cheaper to ship if a smaller step is preferred.

## Evidence traceability

Currently-exists claims trace to: Box-code/lib/lock_controller.py, lock_servo.py,

lock_battery.py, lock_settings.py, lock_ui.py, lock_power.py, lock_config.py,

Box-code/code.py, Box-code/safemode.py.

Unused-radio claim: grep of Box-code/ finds no wifi / bleio / socketpool /

adafruit_requests usage.

SD (sdioio/sdcardio) and BLE (_bleio) are capabilities to build and verify in this

board's CircuitPython 10.2.1 build, not existing code.

No firmware was changed. There is no host test suite; any implementation needs on-device

verification.

## Sources

Board / SD slot & BLE 5: Waveshare ESP32-S3-Touch-LCD-1.47 product page

CircuitPython BLE on ESP32-S3: _bleio docs

CircuitPython SD interfaces: sdioio docs, sdcardio docs

ESP32-S3 native 4-bit SD/MMC host: ESP-IDF SDMMC driver

Android notification access: NotificationListenerService (Android Developers)

iOS notification restriction + 2026 change: Apple third-party notification access rules (9to5Mac), iOS 26.3 notification forwarding (MacRumors)

Competitors: Brick, Unpluq, Unpluq Family, Kitchen Safe, Habit Control