# Phone Box — Battery Fuel-Gauge Board: Longlist & Shortlist

Classification: BUSINESS CONFIDENTIAL — PROCUREMENT

Date: 2026-07-23 · Prototype quantity, USD. Prices read live 2026-07-23.

Job: supplier-discovery-and-qualification

## The ask, and the objective behind it

Literal ask: find a small board to *accurately* measure the battery percentage.

Objective: an accurate state-of-charge (SoC) readout for function #5 (battery

sensing/display). Today the firmware (Box-code/lib/lock_battery.py) reads VBAT

through the incumbent board's 200K/100K divider on GPIO12 and maps volts→percent

with a fixed curve (batt_pct() in lock_config.py). That is inherently

inaccurate on a single LiPo:

The LiPo discharge curve is flat from ~30–80%, so a few mV of noise swings

the displayed % by 10–20 points.

Voltage sags under load — the servo actuation pulse and the backlight both

drop VBAT momentarily, so % reads low mid-action, then jumps back.

The curve shifts with temperature and cell age, and the code already has to

fudge a BAT_CHG_COMP = 0.12 V offset while charging to stop it over-reading.

A dedicated fuel-gauge IC fixes this in hardware: it runs a battery model

(OCV relaxation, or OCV + coulomb counting) and outputs a compensated %.

> Zero-cost alternative, stated honestly (attack-the-objective check): the

> incumbent Waveshare board has no gauge IC — only the divider. Some of the

> inaccuracy could be reduced in firmware alone (rest-voltage OCV lookup, ignore

> readings during a servo pulse, light coulomb integration) for $0 BOM. That

> will *not* match a real gauge under the pulsed servo load, but it is the correct

> baseline to weigh a ~$3–6 part against. Recommendation below still favors the

> board, because the ask is for accuracy and the part is cheap and tiny.

## Search brief (Phase 1)

| Field | Value |
| --- | --- |
| Business outcome | Accurate battery-% display (function #5), robust to servo/backlight load |
| Spend category | Commodity electronic component / breakout module, prototype qty |
| Must-haves | Single-cell (1S) LiPo gauge; CircuitPython library exists (project rule); I2C; small module; better SoC than a voltage curve |
| Strong-preference | I2C so it can share the existing touch I2C bus (no new GPIO); STEMMA-QT/Qwiic plug for solderless bring-up |
| Constraints | CircuitPython runtime only; respect pin map (avoid strapping GPIO0/3/45/46; in use: GPIO12 batt, 41/42/47/48 touch, 13–18 SD, 1/5/10 btn+servo, LCD) |
| Geography | US shipping |
| Disqualifiers | No CircuitPython driver; multi-cell-only; needs a sense resistor we can't place |

Pin-map finding (verified in `lock_config.py`): the AXS5106L touch controller

already runs on an I2C bus (GPIO41/42/47/48). Every gauge below is I2C at a

different address (MAX1704x = 0x36), so it **shares the touch bus and consumes

zero additional GPIO** — a key qualifier for this cramped board.

## Longlist (Phase 3) — 12 candidates by channel

| # | Candidate | Chip / method | Channel | ~Price | CircuitPython | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Adafruit MAX17048 #5580 | MAX17048 · ModelGauge (OCV, no sense R) | First-party maker | $5.95 | ✅ first-party adafruit_max1704x | STEMMA-QT + JST-PH, 25.7×20.3×7.2 mm, addr 0x36 |
| 2 | Adafruit LC709203F #4712 | LC709203F · OCV | First-party maker | $7.50 | ✅ adafruit_lc709203f | Discontinued — chip EOL, Adafruit replaced it with #5580 |
| 3 | 7Semi MAX17048 Breakout | MAX17048 | Reseller (Adafruit-design clone) | ~$4–5 (unconf.) | ✅ (same lib) | Sold via Evelta/Robocraze; STEMMA-QT footprint |
| 4 | 7Semi MAX17048 Mini Breakout | MAX17048 | Reseller | ~$3–4 (unconf.) | ✅ (same lib) | Smallest footprint variant |
| 5 | SparkFun Qwiic Fuel Gauge MAX17048 | MAX17048 | Competing maker | $5.95 | ⚠️ Arduino-first; works via adafruit_max1704x | Retired / out of stock |
| 6 | SparkFun LiPo Fuel Gauge MAX17043 (TOL-20680) | MAX17043 · ModelGauge | Competing maker | ~$11 (unconf.) | ⚠️ via adafruit_max1704x (43 supported) | Qwiic |
| 7 | SparkFun BQ27441 LiPo Fuel Gauge | BQ27441 · coulomb + OCV | Competing maker | retired | ❌ no maintained CP lib (Arduino/Particle only) | Needs sense resistor; higher accuracy, more setup |
| 8 | DFRobot Gravity Li Battery Fuel Gauge (DFR0563) | MAX17043 | Adjacent / education | ~$6–7 (unconf.) | ⚠️ via adafruit_max1704x | Gravity connector = STEMMA/Grove-compatible; screw terminal for cell |
| 9 | HiLetgo MAX17043 module | MAX17043 | Budget marketplace (Amazon) | ~$7 (unconf.) | ⚠️ via adafruit_max1704x | Bare module, no plug connector |
| 10 | Generic MAX17048 module (Amazon/AliExpress) | MAX17048 | Budget marketplace | ~$2–6 | ⚠️ via adafruit_max1704x | Quality/QC variance; verify it's a real Maxim IC |
| 11 | MAX17260 / MAX1726x class (ModelGauge m5) | coulomb + OCV | Component-level | n/a as tidy 1S breakout | ❌ no first-party CP lib | Highest accuracy; needs sense R + config — overkill here |
| 12 | Firmware-only, no board | software OCV/relaxation on existing divider | Incumbent (in-house) | $0 | n/a (already CP) | Baseline to beat; won't match a gauge under load |

Channels represented: first-party maker, reseller of maker design, competing

maker, adjacent/education vendor, budget marketplace, component-level, and the

incumbent (do-nothing/firmware) path.

## Shortlist scorecard (Phase 4)

Weighted criteria, set before scoring. Score 1–5 (5 best); **must-haves gate

before price** (per skill guardrail). "unknown" where evidence is missing.

| Criterion | Weight | #1 Adafruit MAX17048 | #6 SparkFun MAX17043 | #8 DFRobot DFR0563 | #10 Generic MAX17048 | #12 Firmware-only |
| --- | --- | --- | --- | --- | --- | --- |
| Technical fit (1S, I2C, shares touch bus) | 25% | 5 | 5 | 5 | 5 | 3 |
| CircuitPython support (must-have) | 25% | 5 (first-party lib) | 4 (same lib, 43 tested) | 4 (same lib) | 4 (same lib) | 5 (already CP) |
| Accuracy method | 15% | 5 (ModelGauge) | 5 (ModelGauge) | 5 (ModelGauge) | 5 (ModelGauge) | 2 (voltage curve) |
| Bring-up ease (plug + docs) | 15% | 5 (STEMMA-QT + Adafruit guide) | 4 (Qwiic) | 4 (Gravity/screw term) | 2 (bare, no plug) | 5 (no wiring) |
| Availability / US stock | 10% | 5 (in stock) | 3 (unknown) | 4 (unknown) | 4 | 5 |
| Commercial / price | 10% | 4 ($5.95) | 2 (~$11) | 3 (~$6–7) | 5 (~$2–6) | 5 ($0) |
| Weighted total |  | 4.85 | 4.05 | 4.25 | 4.05 | 3.75 |

Notes on scoring:

CircuitPython is the gating must-have. The adafruit_max1704x library

supports both MAX17048 and MAX17043, so every Maxim-based module (rows

1,3,4,5,6,8,9,10) is drivable with the *same* first-party driver — a big

qualification advantage for the whole MAX170xx family and against the BQ27441

(row 7, no maintained CP lib) and the LC709203F (row 2, discontinued chip).

Accuracy is scored on *method*: ModelGauge/OCV (no sense resistor) beats the

raw voltage curve; coulomb-counting parts (BQ27441, MAX1726x) would score even

higher on paper but fail the CP-library / setup gates for a prototype.

## Exclusions & risk register (Phase 5)

Excluded (documented so they're not re-litigated):

| Candidate | Reason |
| --- | --- |
| #2 Adafruit LC709203F | Chip is EOL (Onsemi, 2021) and the board is discontinued/replaced by #5580. Don't design in a dead part. |
| #5 SparkFun Qwiic MAX17048 | Retired / out of stock — same chip as #1, no availability advantage. |
| #7 SparkFun BQ27441 | No maintained CircuitPython library (Arduino/Particle only) → fails must-have. Also needs a sense resistor + config: more accuracy than this app needs, more work. |
| #11 MAX17260 / MAX1726x | No first-party CP driver; requires sense resistor + model config. Overkill for a mostly-sleeping lockbox. |

Risk register:

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| I2C address/timing clash with AXS5106L touch on the shared bus | Low | Med | MAX1704x = 0x36, distinct from touch; scan bus at bring-up; both are 3.3 V logic |
| Firmware rework: replace lock_battery.py ADC path with I2C read | Med | Low | adafruit_max1704x gives .cell_voltage + .cell_percent directly; drop-in swap for Battery.read(), keep the same BatteryReading shape |
| Physical mount / wiring of an extra module in the enclosure | Low | Low | Module is ~26×20 mm; prior CAD check confirmed ~196×195×27 mm envelope has ample room. Tap SDA/SCL/3V3/GND (4 wires) — no new GPIO |
| Generic (#10) module is a counterfeit/re-marked IC | Med | Med | Only if going budget: buy 1, verify addr 0x36 + sane readings before committing |
| Charge-detect: gauge reports SoC, not "charging" flag | Low | Low | Keep existing supervisor.runtime.usb_connected for the charging indicator; gauge supplies %/voltage |
| Adds a line item vs. the current cost-down goal (~+$5.95) | — | Low | It's an *accuracy* upgrade, not a cost cut; if % accuracy isn't worth ~$6, take the $0 firmware path (#12) instead |

## Recommendation & next step

Primary: Adafruit MAX17048 (#5580), $5.95. Best-scoring, in stock in the US,

first-party CircuitPython library (adafruit_max1704x), ModelGauge SoC with **no

sense resistor**, STEMMA-QT plug for solderless bring-up, tiny (26×20 mm), and it

shares the existing touch I2C bus — zero new GPIO. This is the accurate,

low-risk answer to the literal ask.

**Budget variant (if the ~$6 matters at volume): a generic MAX17048/MAX17043

module (#10, ~$2–6) or the 7Semi mini (#4)** — same chip, same

adafruit_max1704x driver — but buy one and verify it's a genuine Maxim IC first.

Decision gate before buying: the ask is *accuracy*, and a fuel gauge adds

~$5.95 while the current cost priority is BOM reduction. If a firmware-only OCV

improvement (#12, $0) gives "good enough" %, that may be the better call. I can

prototype that firmware change first (free) so you can see how close software

alone gets before spending on the board.

Recommended action: buy one Adafruit MAX17048 (#5580) for a bench prototype —

confirm the shared-bus read alongside the AXS5106L touch and validate the

lock_battery.py swap — before committing the part to the BOM. (RFI/RFQ not

needed at $6 prototype qty.)

*Every price/spec claim is backed by a source read live 2026-07-23. Items marked

"unconf." lacked a clean price fetch and are engineering estimates — confirm before

ordering.*

### Sources

Adafruit MAX17048 #5580 — price/size/specs: https://www.adafruit.com/product/5580

Adafruit MAX17048 CircuitPython guide: https://learn.adafruit.com/adafruit-max17048-lipoly-liion-fuel-gauge-and-battery-monitor/python-circuitpython

adafruit_max1704x library (MAX17048 and MAX17049/43): https://github.com/adafruit/Adafruit_CircuitPython_MAX1704x

LC709203F EOL / replaced by MAX17048: https://learn.adafruit.com/adafruit-lc709203f-lipo-lipoly-battery-monitor/python-circuitpython and https://www.adafruit.com/product/4712

SparkFun Qwiic Fuel Gauge MAX17048 (retired): https://www.sparkfun.com/qwiic-fuel-gauge-max17048.html

SparkFun MAX1704X hookup (accuracy ±7.5 mV, hibernation): https://learn.sparkfun.com/tutorials/lipo-fuel-gauge-max1704x-hookup-guide/all

SparkFun MAX1704x Arduino library (covers TOL-20680 MAX17043 + SPX-17715): https://github.com/sparkfun/SparkFun_MAX1704x_Fuel_Gauge_Arduino_Library

SparkFun BQ27441 (coulomb counter, Arduino/Particle lib only): https://docs.particle.io/reference/device-os/libraries/s/SparkFun_BQ27441/

DFRobot Gravity 3.7V Li Battery Fuel Gauge DFR0563 (MAX17043): https://wiki.dfrobot.com/dfr0563/

7Semi MAX17048 breakouts: https://7semi.com/max17048-lipoly-liion-fuel-gauge-mini-breakout/

HiLetgo MAX17043 (Amazon budget module): https://www.amazon.com/HiLetgo-MAX17043-Lithium-Battery-Converter/dp/B01NBE99EP