# Board Alternative — Supplier Longlist & Shortlist

Classification: BUSINESS CONFIDENTIAL — PROCUREMENT

Date: 2026-07-19 · Job: supplier-discovery-and-qualification

Goal: Cut the dominant Phone Box BOM line — the ~$25 Waveshare ESP32-S3

Touch LCD 1.47" board (~62% of the $40.77 per-prototype BOM) — while preserving

all 7 functions and staying on CircuitPython.

# Requirement set (qualification criteria)

Must-haves (a miss disqualifies):

1. CircuitPython-supported MCU with a working displayio build.

2. Integrated color display, usable rectangular area ≈172×320 or larger.

3. Capacitive touch on the display (the swipe-set timer depends on it).

4. Free PWM GPIO for the servo.

5. LiPo charging + a battery-voltage ADC path (onboard, or addable).

6. ≥3 free GPIO (2 buttons + override) and NVM for settings.

Cost angle: firmware uses no Wi-Fi/BLE, so a radio-free MCU pays for nothing

unused. Geography: US hobby/prototype channels. Pricing basis: prototype quantity.

# Incumbent

Waveshare ESP32-S3-Touch-LCD-1.47 — 172×320, AXS5106L capacitive touch,

Wi-Fi/BLE (unused), official CircuitPython waveshare_esp32_s3_touch_lcd_1_47.

$24.99 (Amazon B0F8B8JF74).

# Longlist (14 candidates, by channel)

| Board | Channel | MCU (radio) | Display | Touch (cap?) | LiPo chg / batt sense | CircuitPython | Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sunton ESP32-2432S024C (2.4") | budget vendor | ESP32 (WiFi/BLE) | 2.4" 240×320 ILI9341 SPI | CST820 (yes) | no / no | official build | ~$13–18 |
| Sunton ESP32-2432S032C (3.2") | budget vendor | ESP32 (WiFi/BLE) | 3.2" 240×320 SPI | GT911 (yes) | no / no | official build | ~$15–20 |
| Sunton ESP32-2432S028R (CYD) | budget vendor | ESP32 (WiFi/BLE) | 2.8" 240×320 ILI9341 | XPT2046 (resistive) | no / no | official build | ~$10–15 |
| LILYGO T-Display-S3 Touch | OEM-direct | ESP32-S3 (WiFi/BLE) | 1.9" 170×320 ST7789 | CST816 (yes) | TP4054 / IO4 | official build | $20–36 (volatile) |
| Waveshare RP2040-Touch-LCD-1.28 | OEM-direct | RP2040 (no radio) | 1.28" round 240×240 | CST816S (yes) | yes / unconfirmed | official build | $22.99 direct |
| Waveshare RP2350-Touch-LCD-1.28 | OEM-direct | RP2350 (no radio) | 1.28" round 240×240 | CST816S (yes) | yes / yes (demo) | official build | ~$24–27 |
| Waveshare ESP32-S3-Touch-LCD-2 | OEM-direct | ESP32-S3 | 2.0" 240×320 | CST816D (yes) | yes / — | official build | $25.99 |
| Guition JC2432W328C | budget vendor | ESP32 | 2.8" 240×320 ST7789 SPI | CST820 (yes) | unconfirmed | no build; bring-up feasible | ~$13–18 |
| Guition JC3248W535C | budget vendor | ESP32-S3 | 3.5" 320×480 QSPI | AXS15231B (yes) | onboard chg | none (QSPI unsupported) | ~$17 |
| Elecrow CrowPanel 2.4/2.8" | budget vendor | ESP32 | 240×320 ST7789 SPI | varies | onboard chg | no build; bring-up feasible | $16–25+ |
| Waveshare RP2350-Touch-LCD-2.8 | OEM-direct | RP2350 (no radio) | 2.8" 240×320 | 5-pt cap (yes) | yes / — | no build (custom def) | $28.95 |
| Adafruit ESP32-S3 Reverse TFT Feather | reference | ESP32-S3 | 1.14" 240×135 | none | yes / MAX17048 | first-class | $24.95 |
| Adafruit PyPortal | reference | SAMD51+ESP32 | 3.2" 320×240 | resistive | no / no | first-class | $54.95 |
| Pimoroni Presto | OEM-direct | RP2350B (WiFi) | 4" 480×480 | cap (yes) | no / no | none (MicroPython) | ~$74 |

# Shortlist scorecard

Weights: technical fit (cap touch + rect screen + PWM) 35% · CircuitPython quality

25% · cost reduction 20% · battery integration 12% · supply risk 8%.

| # | Board | Price | Cost outcome | Verdict |
| --- | --- | --- | --- | --- |
| 1 | Sunton ESP32-2432S024C | ~$15 | board line ~$25 → ~$16.5 all-in (adds ~$1.5 charger/divider) → saves ~$8–9/unit (~20% of BOM) | Primary. Official CircuitPython, CST820 capacitive, 240×320 (larger than incumbent), free PWM. Only gap = no onboard battery hardware. |
| 2 | Sunton ESP32-2432S032C | ~$18 | saves ~$5–6/unit | Backup; larger 3.2" GT911. |
| 3 | LILYGO T-Display-S3 Touch | ~$37 (confirmed) | not a cost win | Dropped: live US price $36.95–38.95 (ProtoSupplies) and sold out at lilygo.cc US — dearer than the incumbent. |
| 4 | Waveshare RP2350-Touch-LCD-1.28 | ~$24 | ~$1–3 saving, radio-free | Only if radio-free is prioritized over UI; 1.28" round screen is a real UI-fit risk. |

# Purchase links (checked 2026-07-19)

Only prices read live on the listing are quoted as exact; Amazon/waveshare.com

block price scraping (URL confirmed to resolve to the right product, price ~typical).

| Board | Buy link(s) | Price | Stock / note |
| --- | --- | --- | --- |
| Sunton ESP32-2432S024C (2.4" cap) | Amazon US — DIYmalls | ~$15 (not live-readable) | Confirm it's the C (capacitive) SKU |
| Sunton ESP32-2432S032C (3.2" cap) | Amazon US — DIYmalls | ~$18 (not live-readable) | Capacitive IPS confirmed |
| Waveshare RP2350-Touch-LCD-1.28 | The Pi Hut · Waveshare | £18.30 (~$23) live | In stock; round screen |
| Waveshare RP2040-Touch-LCD-1.28 | The Pi Hut · Waveshare | £22.10 (~$28) live | ~16 units left |
| LILYGO T-Display-S3 Touch | ProtoSupplies (US) · lilygo.cc | $36.95–38.95 live | Not a cost win — dearer than incumbent; sold out at lilygo US |
| Incumbent ESP32-S3-Touch-LCD-1.47 | The Pi Hut · Amazon US | £14.40 (~$18–19) live @ Pi Hut vs ~$25 Amazon | See note below |

Cheap win, zero redesign: the *same* incumbent board is £14.40 (~$18–19) at

The Pi Hut vs. the ~$25 Amazon basis — sourcing the current board from a cheaper

channel saves ~$6/unit with no engineering work. Confirm US shipping/duty on the order.

# CAD check — can a larger battery fit?

Measured from the STEP files (units = mm; bounding box of part geometry):

Main Case: ≈ 196 × 195 mm footprint, wall height ~27 mm (usable interior

depth ~20–25 mm after floor/lid clearance), interior footprint ~190 × 190 mm after walls.

Lid: ≈ 130 × 27 mm raised section (closes over the case).

Conclusion: space is not the constraint. The current 1000 mAh cell (~50×34×5 mm)

uses a tiny fraction of a ~190×190 mm floor. A phone occupies only part of that

footprint, leaving large free area beside it. A much larger LiPo fits easily — the

only tight axis is thickness (~15–20 mm of depth realistically available after the

phone + servo/latch mechanism), and high-capacity flat cells are typically 6–12 mm

thick. So capacity can grow substantially (roughly 2000–5000 mAh, e.g. a 606090 /

~4000 mAh pack) on space alone. Practical limits become the board's charge current,

cost, and weight — not the enclosure.

*Caveat:* this is envelope-level from the STEP point geometry; I can't see the exact

internal pocket, phone cradle, or servo clearance without SolidWorks. Confirm the free

pocket beside the phone and the servo swing before finalizing a specific cell.

# Exclusions (do not re-litigate)

Sunton 2432S028R — resistive touch fails the swipe UI.

Guition JC3248W535 / VIEWE AMOLED — QSPI displays; CircuitPython displayio has no QSPI support.

Guition JC2432W328C / Elecrow CrowPanel — no official CircuitPython build (custom bring-up); fallback only.

Waveshare ESP32-S3-Touch-LCD-2 ($26), RP2350-2.8 ($29, no build), Adafruit Reverse Feather ($25, no touch), PyPortal ($55), Pimoroni Presto ($74, no CircuitPython) — fail cost and/or a must-have.

# Risk register (recommended path: Sunton 2432S024C)

| Risk | Severity | Mitigation |
| --- | --- | --- |
| Free-GPIO scarcity on CYD boards | High | Prototype must confirm ≥4 free pins (servo PWM + 2 buttons + override) and a free ADC before committing. |
| No onboard LiPo charge/battery sense | Medium | Add external TP4056 + ~3:1 divider (~$1.50); re-implement USB-power detection. |
| Touch driver port AXS5106L → CST820 | Low | CircuitPython CST8xx libs exist; repoint lock_controller.py gesture logic. |
| UI re-layout 172×320 → 240×320 | Medium | Rework lock_ui.py layout constants (more area available). |
| Budget-vendor revision/pin drift | Medium | Buy from a named reseller; lock SKU/revision. |

# Recommended next step

Buy one Sunton ESP32-2432S024C (~$15) for a bench prototype/demo to validate

free-GPIO count, displayio + CST820 touch bring-up, and the battery add-on

before switching the design. This is a technical-validation demo, not an RFQ

(off-the-shelf part, prototype quantity).

# Sources

Incumbent price: Amazon B0F8B8JF74 · CircuitPython waveshare_esp32_s3_touch_lcd_1_47

Sunton 2432S024C: circuitpython.org board · 2432S032C: board · 2432S028R: board · Random Nerd CYD

LILYGO T-Display-S3: CircuitPython · Touch specs (espboards) · lilygo.cc

Waveshare RP2040-1.28: CircuitPython · RP2350-1.28: CircuitPython · ESP32-S3-Touch-LCD-2: CircuitPython

Guition JC3248W535 (QSPI/no CircuitPython): atomic14 · JC2432W328: maxpill/JC2432W328

Adafruit Reverse TFT Feather: product 5691 · PyPortal: 4116 · Pimoroni Presto: shop

Note on prices: Amazon/reseller figures read live 2026-07-19 and fluctuate;

waveshare.com/lilygo.cc/pimoroni.com JS-render or block automated fetch, so some

direct prices are inferred from resellers — confirm direct pricing before purchase.