# Phone Box — Bill of Materials

Date: 2026-07-23 · Prototype quantity, USD.

Two sections: Current materials (as-built) and Battery meter (fuel-gauge

add-on for accurate battery %).

Board decision — keep the original board. We are staying on the

Waveshare ESP32-S3-Touch-LCD-1.47. It gives us two things we now need and

don't want to rebuild: onboard LiPo charging + battery-sense (charges over

USB-C, no separate charger part) and the BLE radio the companion app talks

to. The earlier Sunton board-swap cost-reduction proposal is therefore dropped:

Sunton has no onboard charging (would need a TP4056 + divider added back) and

swapping it carries a full case + firmware redesign — for only a marginal board

saving. Board price basis from the

supplier shortlist

(read live 2026-07-19). Lines marked *(est.)* are engineering estimates, not

quotes — confirm before ordering.

## Current materials

As-built BOM on the retained Waveshare board (~$43.77 per prototype at retail

pricing).

| # | Item | Spec / part | Qty | Unit $ | Line $ | Source |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | MCU + display board | Waveshare ESP32-S3-Touch-LCD-1.47 — 172×320 cap touch, onboard LiPo charge + batt-sense, BLE, MX1.25 conn | 1 | 24.99 | 24.99 | sourced |
| 2 | Battery | LiPo 2000 mAh 3.7 V, 103450 (~10×34×50 mm), MX1.25 lead | 1 | 10.95 | 10.95 | est. |
| 3 | Lock actuator | Hobby servo (SG90 class), PWM on GPIO5 | 1 | 3.49 | 3.49 | est. |
| 4 | Buttons / switch | Momentary switch ×2 (lock GPIO1 + override GPIO10) | 2 | 0.50 | 0.99 | est. |
| 5 | Enclosure | 3D-printed Main Case + Lid (filament) | 1 | 2.53 | 2.53 | est. |
| 6 | Wiring / fasteners | JST leads, screws, standoffs, misc | 1 | 0.82 | 0.82 | est. |
|  |  |  |  | Total | $43.77 |  |

> Charging is integrated on the board (firmware reads VBAT via the board's

> 200K/100K divider on GPIO12; charges over USB-C). No separate charger line

> needed. The 3rd case cutout = USB-C access to this board, not a standalone

> charger module.

Optional cost-down (board retained — no redesign): these levers still apply

now that the board is fixed. Buying the *same* board from The Pi Hut (~$18.50)

instead of Amazon ($24.99) saves ~$6.49/unit and keeps the onboard charger + BLE.

Bulk-sourcing the est. lines at 10–50 qty (servo → ~$1.50, buttons → ~$0.08 ea,

cell → ~$3.50, wiring → ~$0.50) saves ~$6/unit. Optimizing the print

(15–20% infill, PLA) saves ~$1.20/unit. Moving the lock button on-screen drops

a switch + a case cutout (keep the physical override button for safety).

Together these land the unit at ≈ $28/unit with zero CAD or firmware

redesign. Battery capacity is a runtime/cost dial: the default 2000 mAh 103450

cell can drop to a bulk 1000 mAh (~$3.50) to save ~$7/unit if measured

runtime allows — pick capacity by measured runtime need.

## Battery meter — MAX17048 fuel gauge (optional add-on)

*Full analysis: battery-fuel-gauge shortlist.*

Problem: the current build doesn't measure battery percentage *accurately*.

It reads VBAT through a resistor divider and maps voltage → % with a static

curve (BAT_CURVE in lock_config.py). The LiPo discharge curve is flat across

~30–80% SoC and servo/backlight load sags the voltage, so the displayed % swings

by ~±10–20% and reads low under load. No firmware tweak removes this — it is a

hardware limitation of voltage-only sensing.

Fix: add a MAX17048 fuel-gauge board — a ModelGauge IC that fuses coulomb

tracking + voltage + temperature and reports state-of-charge directly (~±1–2%),

with no sense resistor. Talks I2C at 0x36 (shares the existing touch bus or a

spare busio.I2C; frees GPIO12). CircuitPython support via adafruit_max1704x

(monitor.cell_percent). At 25.7×20.3×7.2 mm it drops into the battery bay.

| Option | Part | Unit $ | Note |
| --- | --- | --- | --- |
| Ready breakout | Adafruit MAX17048 #5580 (JST-PH in/out + Qwiic) | 5.95 | sourced; also Pi Hut / Pimoroni |
| Cheaper breakout | 7Semi MAX17048 mini | ~2–4 (est.) | no Qwiic connectors |
| Cheapest board | Generic MAX17048 module (marketplace) | ~2 (est.) | QC / lead-time risk |
| Volume | Bare MAX17048 IC on custom board | ~1–2 (est.) | production only |

> This is an accuracy upgrade, not a cost cut — it *adds* ~$2–6/unit. Take it

> only if accurate % is a product requirement; otherwise the $0 fallback is a

> firmware-only OCV improvement (smoothing + settle-when-idle), which helps but

> cannot fix load sag or the flat curve. Replaces LC709203F as the modern pick

> — that chip is EOL (discontinued).

## Notes & caveats

2000 mAh fits the enclosure (current case): 103450 (~10×34×50 mm) drops into

the clear bay window between the charger cutout (ends Z≈53) and the screen

standoffs (start Z≈112): ≈53 L × 30 W × 18 T mm available, cell uses ~10 mm depth.

Cheaper zero-redesign board sourcing: the *same* incumbent board is ~$18–19

at The Pi Hut vs. $24.99 Amazon — sourcing switch alone saves ~$6/unit with no

engineering. Confirm US shipping/duty.

Est. lines (battery, servo, buttons, enclosure, wiring) are placeholders —

replace with real quotes before an RFQ.