# Phone Box Companion App — Development Approach (Technical Design)

Date: 2026-07-22

Type: RFC / technical design (research-grade recommendation, pre-implementation)

Prepared for: Phone Box (CircuitPython touchscreen phone-lock box, Waveshare ESP32-S3-Touch-LCD-1.47)

Upstream context: docs/brainstorming/phone-box-feature-ideation-2026-07-20.md — established the companion

app as the keystone premium feature (the ESP32-S3 radio is in the BOM but unused). That document

answered *what* to build and *why*. This document answers *how* to build it: connectivity shape,

firmware GATT design, mobile framework selection, MVP scope, validation, and risks.

## Executive summary — the recommendation

Build the companion app as a BLE-first, offline, local app, with a **cross-platform React Native

codebase on the phone and a adafruit_ble GATT peripheral in the firmware. Ship a read-only

"viewer" MVP** (status + session history + basic settings) before adding write-path features

(scheduling, greenlist unlock, accountability). Defer any Wi-Fi/cloud backend until the

accountability/subscription tier is actually validated — it is the only part that carries permanent

cost and privacy liability.

Three decisions drive everything below:

Transport: BLE, not Wi-Fi/cloud (at first). BLE needs zero backend, works offline, is private,

and the phone supplies real timestamps. It maps cleanly onto the box's existing local-only,

no-account model and adds $0 BOM (radio already present).

Firmware: `adafruit_ble` over raw `_bleio`. Adafruit explicitly recommends the higher-level

library; the ESP32-S3 has native BLE and CircuitPython 10.2.1 supports full GATT peripheral/server

with custom services and read/write/notify characteristics.

App: React Native + `react-native-ble-plx`. The box's interaction is exactly the pattern

cross-platform BLE libraries handle best — *connect, read, write, subscribe-to-notify* — so one

codebase serves iOS and Android. The one feature that genuinely needs native code (greenlist call

unlock) needs native code in *any* framework, so it is not a reason to go fully native for the whole

app.

Greenlist is iOS-first, via calls (per manager direction). The prioritized greenlist behavior is

*a whitelisted contact's call reaches through the locked box*. iOS is now the lead platform for it —

see §5.3 for exactly what iOS allows (an any-call "someone's calling" alert via CXCallObserver, and

true per-contact greenlisting when the call routes through the app as VoIP/PushKit, which also

gives iOS a *reliable* background wake — better than CoreBluetooth state restoration).

Two build options (BOM, §2a). Because the radio is already on the original board, the app adds

$0 hardware. A "without Bluetooth" (radio-free, no-app) build does not save money at prototype

quantity — the cheapest radio-free CircuitPython touch boards are *dearer* than the original board and

have a worse screen. Keeping the original board is both cheaper and app-capable.

Confidence: 75/100 that this approach ships the viewer MVP successfully. The uncertainty is not the

architecture (well-trodden) but two unverified board facts and the platform limits on iOS call

greenlisting — both scoped in §5.3 and the risks below.

## 1. Customer & problem

Customer: buyers who already failed with free Screen Time / app blockers and are paying $59+ for

*physical* friction, plus the accountability/family segment (the only validated recurring-revenue tier

in this category, per the ideation doc's competitor research).

Problem the app solves: today the box captures no data a user can see, offers only a single manual

countdown, and has no remote/scheduled/accountability behavior. Competitors (Brick, Unpluq, kSafe) win

on exactly these software capabilities. The box hardware already exceeds them; the gap is software.

What the app must let a user do (in eventual scope):

See the box's live state and battery, and their focus history / streaks (the dashboard that justifies

the price band).

Start / configure a lock and set recurring schedules (e.g. 10pm–7am nightly).

Manage settings from the phone instead of the tiny on-device UI.

(Later) accountability: a partner/family member can see sessions and gate the override.

(Later) greenlist: a whitelisted call/notification can alert-through or release the box.

## 2. Requirements grounded in the current firmware

The app is a second interface onto the existing state machine, not a rewrite. The integration

seams already exist:

| Need | Existing firmware anchor (Box-code/lib/…) |
| --- | --- |
| Box state to report | lock_controller.py state = idle / closed / running / done; deadline, set_seconds |
| Start a lock from the app | go_running(now) (same path the on-screen LOCK button calls) |
| Release early from the app | release_lock() + go_done(now) (same path the override button reaches) |
| Session record to log/stream | the go_done transition is the natural emit point (completed vs overridden, set_seconds, override count) |
| Settings to read/write | lock_settings.py — override_presses, auto_open, sleep_s, bright_pct, persisted in NVM |
| Battery to report | lock_battery.py Battery.read(now) |
| Wall-clock time (box has none) | phone pushes time over BLE; box has countdown only (no RTC, resets on power loss) |

Hard constraints (from `project_context.md` / `project_rules.md`):

CircuitPython only — no CPython/pip deps; only modules in the CP 10.2.1 runtime + vendored libs.

lock_config.py is the single source of truth for pins/tunables; respect the pin map (avoid

strapping pins, and GPIO12 battery, 41/42/47/48 touch, 13–18 SD, LCD pins).

Keep the run loop responsive: touch is read before the heavier redraw; BLE servicing must not

block or reorder that. CPU-frequency scaling must not interrupt an active touch/servo interaction.

No host build/test — validation is on-device only. Deploy = batch-write all files then sync;

a read-only D: means FAT corruption.

Deliverables are Word docs.

## 2a. Build options & BOM — connectivity vs. radio-free

The manager asked for two costed build options: (A) without Bluetooth and **(B) the original

board. The decisive fact from the prior board-cost work: the app needs** the ESP32-S3 radio, and

that radio is *already on the original board* — so the app is $0 additional hardware. Going

radio-free is a *different product* (no app is possible) and it is not cheaper at prototype quantity.

Both BOMs share the same non-board lines (from docs/procurement/bom.md, prototype qty, USD; (est.)

lines are engineering estimates, not quotes): 1000 mAh LiPo $7.95, servo $3.49, buttons ×2 $0.99,

enclosure $2.53, wiring/fasteners $0.82 → $15.78 shared.

### Option A — Without Bluetooth (radio-free, app not possible)

No off-the-shelf radio-free CircuitPython capacitive-touch board beats the original at prototype

quantity. The closest are Waveshare's RP2040/RP2350-Touch boards — but they are more expensive and

use a 1.28" round screen that does not fit the swipe-to-set UI (a UI-moat risk flagged in the board

shortlist).

| Line | Part | Unit $ | Link |
| --- | --- | --- | --- |
| Board (radio-free) | Waveshare RP2350-Touch-LCD-1.28 (1.28" round, no radio) | ~23.00 | The Pi Hut · Waveshare · CircuitPython |
| Shared lines | battery + servo + buttons + enclosure + wiring | 15.78 | see bom.md |
| Total |  | ≈ $38.78 |  |

*Caveats:* round 172-equivalent area is smaller and non-rectangular (UI rework + moat risk); a

radio-free build cannot run the companion app, scheduling, greenlist, accountability, or OTA. The

alternative RP2040-Touch-LCD-1.28 is ~$28 (Pi Hut),

i.e. still dearer. Radio-free only becomes a saving at custom production volume (spec a board without

the radio), not at prototype quantity.

### Option B — Original board (ESP32-S3, app-capable, $0 app hardware)

| Line | Part | Unit $ | Link |
| --- | --- | --- | --- |
| Board (has BLE) | Waveshare ESP32-S3-Touch-LCD-1.47 (172×320 cap touch, onboard LiPo charge + batt-sense) | 18.50 (Pi Hut) – 24.99 (Amazon) | The Pi Hut ~£14.40/$18–19 · Amazon $24.99 · CircuitPython |
| Shared lines | battery + servo + buttons + enclosure + wiring | 15.78 | see bom.md |
| Companion app | uses onboard Wi-Fi/BLE — no added part | 0.00 | — |
| Total |  | ≈ $34.28 (Pi Hut board) – $40.77 (Amazon board) |  |

### Comparison

|  | A — Without Bluetooth | B — Original board |
| --- | --- | --- |
| Board cost | ~$23 (RP2350) | $18.50–24.99 (ESP32-S3) |
| BOM total | ~$38.78 | ~$34.28–40.77 |
| Companion app / scheduling / greenlist / accountability / OTA | impossible | enabled, $0 hardware |
| Screen | 1.28" round (UI-moat risk) | 172×320 rectangular (current UI) |
| Redesign | firmware + enclosure rework | none |

Recommendation: keep the original board (Option B), sourced from The Pi Hut (~$18.50) rather than

Amazon (~$24.99) for a ~$6/unit no-engineering saving. It is both **cheaper than the radio-free option

and the only one that can run the app**. Ship the app as a *software upgrade on the existing hardware*,

not a new SKU. (If a truly basic no-app SKU is ever wanted, get it by *not shipping the app* on the same

board — not by paying more for a radio-free board.)

## 3. Architecture overview

### 3.1 Transport options (decision: start at B1)

Reproduced and sharpened from the ideation doc's Deep-dive A, now as an architecture decision:

| Option | Shape | BOM | Ongoing cost | Effort | Offline | Real time | Monetization fit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1 (chosen for MVP) | BLE peripheral ↔ phone app, no internet | $0 | none | medium | yes | from phone | strong (Brick-tier) |
| B2 | Wi-Fi, box hosts local web page | $0 | none | medium | LAN only | NTP | weak ("not a product") |
| B3 (defer) | Wi-Fi + cloud backend, accounts | $0 | backend forever | high | no | NTP | strongest (subscription) |

Decision: B1 for the MVP and the scheduling/greenlist tiers. B3 only when the accountability

subscription is validated enough to justify a permanent backend, accounts, security surface, and

privacy burden. B2 is skipped — it has B3's "join a network" friction without B3's payoff.

### 3.2 Data-capture seam (build once, feed both sinks)

Per the ideation doc's Deep-dive A recommendation, add one "emit a session record" seam at the

lock_controller.py go_done transition. Two independent sinks consume it:

SD log (sdioio/sdcardio) — cheap, robust local source-of-truth; survives when the phone is

absent; no CIRCUITPY/FAT write conflict (external card mounted from code.py).

BLE stream — the app reads the same records live and/or backfills on connect.

Building SD logging *first* de-risks the app: the app becomes a *viewer/sync layer* over data that

already exists, rather than the thing responsible for capturing it.

### 3.3 End-to-end flow (MVP)

Box (ESP32-S3, CircuitPython)                     Phone (React Native app)
┌──────────────────────────────┐                 ┌───────────────────────────┐
│ code.py run loop             │                 │ BLE scan by service UUID  │
│  1 read touch                │   BLE GATT      │  → connect → discover     │
│  2 controller.update()       │◄──────notify────│  → subscribe (status,     │
│  3 controller.process()      │─────read/write─▶│     history)              │
│  4 service_ble()  ← new, nonblocking            │  → render dashboard       │
│ adafruit_ble Peripheral      │                 │  → write: time sync,      │
│  Service: PhoneBox           │                 │     start/config, settings│
└──────────────────────────────┘                 └───────────────────────────┘

## 4. Firmware design (box side)

### 4.1 Library choice

Use `adafruit_ble` (the supported wrapper) rather than raw _bleio. Confirm the board's CP 10.2.1

build includes _bleio/adafruit_ble before relying on it (see Risks). GATT server, custom services,

and read/write/notify are fully supported on ESP32-S3 native BLE; BLE 5 allows an ATT MTU up to ~247

bytes, comfortably above our per-record size (~30–60 bytes).

### 4.2 New module: lock_ble.py (sibling driver, matches existing pattern)

Following the project's per-peripheral driver convention (lock_servo.py, lock_battery.py, …), add a

single lock_ble.py that owns the radio and exposes a non-blocking `service()` called once per run

loop, after touch+update, so it never delays touch sampling.

Proposed GATT service — `PhoneBox` (one custom 128-bit service UUID):

| Characteristic | Props | Payload (packed bytes / short JSON) | Backed by |
| --- | --- | --- | --- |
| status | READ, NOTIFY | state enum, remaining_s, set_s, battery_pct, fw_ver | controller.state, deadline, battery.read() |
| command | WRITE | opcode + args: start(duration), unlock, lock, set_schedule(...) | calls go_running / release_lock+go_done / go_closed |
| history | READ, NOTIFY | session records, newest-first, chunked | the go_done emit seam / SD log |
| settings | READ, WRITE | override_presses, auto_open, sleep_s, bright_pct | lock_settings.Settings (+ save()) |
| time_sync | WRITE | epoch seconds from phone | sets box wall-clock for dated history/schedules |

Notes:

`command` is policy-gated. A BLE unlock is a *new early-release path*; rate-limit it and keep

the physical press-count override as the true emergency path (mirrors the ideation doc's integrity

caveat for greenlist). Default new remote paths to *alert-through*, not auto-unlock.

Pairing/bonding + a pairing screen on the box (lock_ui.py gets one new view) so a random phone

can't drive the latch. Bond so reconnect is silent.

Advertise the service UUID so iOS background scanning can find it (iOS requires scanning by

service UUID in the background).

### 4.3 Integration points (small, additive)

code.py run loop: add ble.service() as a new step after touch read + controller.update() +

controller.process() — preserving the documented ordering rule.

lock_controller.py go_done: emit a session record (the one seam) → SD sink + BLE history

notify.

lock_controller.py: a thin command dispatcher that maps BLE opcodes onto existing transitions

(no new lock mechanism — reuse go_running, release_lock, go_closed).

lock_settings.py: already has save(); BLE settings writes call the same adjust/save path,

and bump _MAGIC if the NVM layout changes.

lock_config.py: add BLE tunables (device name, service UUID, advertise interval, rate-limit)

there, not scattered.

Power note: the radio adds current draw; gate advertising/connection by policy (e.g. advertise only

when idle/closed or when the screen is awake) so battery life and the brownout budget are respected —

the servo already must be powered from VBAT/VBUS with a bulk cap, and BLE TX peaks add to that budget.

## 5. App design (phone side)

### 5.1 Framework selection

Recommendation: React Native (Expo dev build) + `react-native-ble-plx`.

| Option | Pros | Cons | Verdict |
| --- | --- | --- | --- |
| React Native + `react-native-ble-plx` | One codebase iOS+Android; library is mature and covers exactly connect/read/write/notify + background modes + device filtering; largest RN BLE community; Expo config plugin exists | Background BLE + native notification access need native modules/config; RN BLE slightly less rock-solid than Flutter's | Chosen — best effort/coverage fit for this device's simple GATT |
| Flutter + flutter_blue_plus | BLE plugins generally *more stable*; single codebase; good perf | Team must be Dart-fluent; greenlist still needs platform channels to native | Strong alternative; pick if the team already does Flutter |
| Native (Swift + Kotlin) | Best BLE control & background reliability; required anyway for greenlist | ~2× the work (two apps); slowest to MVP | Only if background BLE reliability becomes the dominant risk |

Rationale: the box exposes a handful of characteristics with the canonical *connect → read → write →

subscribe* interaction — the case cross-platform BLE libraries handle well. Custom/proprietary GATT and

heavy background work are what push teams to native; our GATT is simple, and the one background-heavy

feature (greenlist) needs native code in every framework, so it doesn't justify making the *whole* app

native. Start cross-platform; drop to a native module only where the platform forces it.

### 5.2 App architecture

BLE service layer (react-native-ble-plx): scan-by-service-UUID → connect → bond → characteristic

read/write + notify subscriptions; auto-reconnect; a small typed codec for the packed payloads.

State/store: cache last-known status + history locally (e.g. SQLite/MMKV) so the dashboard works

before the box is in range; reconcile on connect.

UI: dashboard (streaks/history), live status, lock/schedule controls, settings mirror, pairing

flow. Use the generic UI baseline (no project design-system configured yet).

Time: phone writes time_sync on every connect so the box has real time for dated history and

schedules.

### 5.3 Greenlisting calls — iOS-first (per manager direction)

The prioritized behavior: a whitelisted contact's phone call should reach through the locked box

(alert-through by default; optional auto-unlock). Manager direction is to lead with iOS, focused on

calls. This is a platform problem, not a framework one, and iOS has a specific, honest boundary that

shapes the design.

What iOS does and does not allow for calls (verified):

| Capability | iOS reality | Use for greenlist |
| --- | --- | --- |
| Detect that *a* call is ringing (no caller ID) | CXCallObserver reports call state (incoming/connected/ended) even in background; it does not expose the caller's number (privacy) | "Someone is calling" alert-through — buzz/screen on the box, no identity filter |
| Identify a specific incoming cellular caller | Not available to third-party apps. CallDirectory/Live Caller ID Lookup extensions only feed labels to Apple's Phone app; they are never told about a live call | ❌ cannot per-contact filter a normal cellular call |
| True per-contact greenlist | Route the call through the app as VoIP (PushKit + CallKit): the app places/receives the call, so it knows the caller and can act | ✅ reliable per-contact greenlist and a reliable background wake |
| Background wake to reach the box | PushKit VoIP push wakes the app reliably; far more dependable than CoreBluetooth state restoration, which is known to fire inconsistently | ✅ use PushKit as the trigger, then connect BLE to signal the box |

iOS design (recommended):

Tier 1 — any-call alert-through (ship first): while the box is locked, CXCallObserver detects an

incoming call and the app signals the box over BLE to alert-through (screen + buzz), *without*

knowing who is calling. Simple, reliable, no backend. Keeps the focus contract (box stays shut).

Tier 2 — true per-contact greenlist via VoIP: the greenlisted contact calls through the app

(a VoIP "priority line", or a relay such as Twilio that rings the app). PushKit wakes the app in

the background → the app confirms the caller → signals the box to alert-through or (opt-in)

auto-unlock via release_lock(). This is the only reliable way to greenlist a *specific* contact on

iOS, and PushKit doubles as the dependable background trigger BLE alone lacks.

BLE background still needs UIBackgroundModes: bluetooth-central + scan-by-service-UUID; but with

PushKit as the primary wake, we do not depend on the flaky CoreBluetooth state restoration for the

call path.

Honesty for product/marketing: iOS can reliably do (a) "your box lights up when *anyone* calls" and

(b) "greenlist a specific person if their call comes through the app." It cannot silently

identify an arbitrary normal cellular call by contact. Do not promise per-contact filtering of ordinary

cellular calls on iOS.

Android (also supported, easier): CXCallObserver's equivalent plus full notification access via

NotificationListenerService (a native module) means Android can greenlist by contact or app

directly, including non-call notifications. Build it after the iOS call path, reusing the same box-side

BLE command and alert-through overlay.

Native code either way: call observation (CallKit/PushKit on iOS, NotificationListenerService

on Android) lives in native modules behind a small JS interface — this is true in React Native,

Flutter, or native, so it does not change the framework choice in §5.1. The box side is unchanged: a BLE

command reaches the existing release_lock() path; default to alert-through, rate-limit, keep the

physical press-count override as the true emergency path.

## 6. MVP scope & phased roadmap

Sequenced to de-risk (cheap/local first, expensive/liability last):

Firmware SD logging + on-device stats view — captures real habit data for ~$3, firmware only,

no app dependency, no FAT-corruption risk. *(This is the data foundation; do it first.)*

MVP app — BLE read-only viewer: pair, live status, history dashboard/streaks, settings

read. This alone justifies the $59–89 band and de-risks all BLE plumbing. *(Chosen MVP.)*

Write path: app-initiated start/lock + time_sync; then scheduled/recurring locks and

multiple profiles (high value once connectivity + time exist).

Accountability / family mode — turns on the validated subscription tier; may pull in B3 cloud.

Greenlist calls — layered on the app; iOS-first (Tier 1 any-call alert-through via

CXCallObserver; Tier 2 per-contact via VoIP/PushKit), then Android. Alert-through by default.

OTA firmware updates over BLE — supports ongoing feature delivery.

## 7. Validation plan (on-device — no host tests exist)

| User scenario | Expected outcome | Validation method |
| --- | --- | --- |
| App scans and finds box | Box advertises PhoneBox service UUID; appears in app | On-device: run app + box, observe scan list |
| Pair/bond | One-time pairing screen on box; silent reconnect after | On-device: pair, kill app, reopen, confirm auto-reconnect |
| Live status | status notify updates remaining_s/battery as countdown runs | On-device: start lock, watch app track the on-screen timer |
| Start lock from app | command:start drives go_running; servo latches | On-device: observe latch + on-screen state = running |
| Early unlock from app | command:unlock (policy-gated) reaches release_lock/go_done | On-device: confirm latch releases; confirm rate-limit blocks spam |
| Session history | A completed/overridden session appears in app + SD log | On-device: finish a session, read app record vs SD CSV |
| Settings round-trip | App write updates NVM; persists across reboot | On-device: change override count in app, reboot box, confirm |
| Time sync | Box gets real time; dated history correct | On-device: connect, check a record's timestamp |
| Run-loop responsiveness | Touch/servo stay snappy with BLE active | On-device: swipe/latch while connected; no stutter |
| Power | Advertising/connection doesn't cause brownout | On-device: run on battery with servo cycling + BLE connected |

No host build/test suite exists; every row above requires a physical board run. State plainly in

any implementation PR when a change is untested because no board run was performed.

## 8. Risks & mitigations

| Risk | Sev | Mitigation |
| --- | --- | --- |
| CP 10.2.1 build may lack `_bleio`/`adafruit_ble` (or `sdioio`/`sdcardio`) | High | Verify on-device before committing to BLE; if absent, this gates the whole track. First spike = import + advertise a trivial service on the real board. |
| iOS CoreBluetooth state restoration is unreliable for background wake | High | Use PushKit VoIP as the background trigger for the greenlist call path (dependable), not state restoration; keep a foreground-connected UX for the viewer. |
| iOS can't identify an arbitrary cellular caller by contact | High | Ship Tier 1 any-call alert-through (CXCallObserver, no identity); do true per-contact greenlist only via VoIP/PushKit (call routed through the app); never promise per-contact filtering of ordinary cellular calls on iOS. |
| BLE `unlock` is a new bypass of the focus contract | Med | Rate-limit; default alert-through not auto-unlock; keep physical press-count as the true emergency path. |
| Radio power draw / brownout budget | Med | Policy-gate advertising; bulk cap; test on battery with servo cycling; this is why cost-down aimed to *remove* the radio — connectivity is a deliberate opposite bet. |
| BLE servicing stalls the run loop / reorders touch | Med | ble.service() is non-blocking and runs *after* touch+update; never inside a touch/servo interaction. |
| Pairing/security — a stranger's phone driving the latch | Med | Require bonding + on-box pairing confirmation; gate command on a bonded connection. |
| Two-tier effort creep (app becomes native anyway) | Low | Keep RN core; isolate native code to the greenlist module behind a clean interface. |
| Cross-platform BLE quirks vs. custom GATT | Low | GATT is deliberately simple (5 characteristics, standard props); stays within library sweet spot. |

## 8a. Architecture analysis

This project has no standalone architecture document; fraim/personalized-employee/context/project_context.md

and project_rules.md are the de-facto architecture record. The design is checked against them:

Patterns correctly followed

Per-peripheral driver module — lock_ble.py mirrors lock_servo.py/lock_battery.py as a

sibling driver; the radio is owned by one module.

`lock_config.py` as single source of truth — all BLE tunables (name, UUID, intervals,

rate-limit) live there, not scattered.

State machine reuse — BLE commands map onto existing go_running/release_lock/go_done; no new

lock mechanism is introduced.

Versioned NVM persistence — settings writes reuse lock_settings.Settings.save() and the magic-

byte bump convention.

Run-loop ordering rule — ble.service() is placed after touch read + update, non-blocking.

On-device-only validation — the validation plan is entirely on-device; no fabricated host tests.

Word-doc deliverable — RFC delivered as .docx via md_to_docx.py.

Patterns missing from architecture (design introduces; needs a doc decision)

Radio/connectivity is explicitly documented as unused in project context. This design

deliberately *adds* it — a stated counter-bet against the cost-down priority. If pursued, project

context should be updated to record connectivity as an intentional up-market feature, not an

oversight. *(User/manager decision required.)*

A new remote early-release path (BLE unlock) and pairing/bonding + security model have no

precedent in the current firmware; they need to be recorded as architecture once accepted.

SD session-logging and a wall-clock time source (phone-supplied) are new subsystems not in

the current architecture.

App-side native-module boundary (iOS CallKit/PushKit, Android NotificationListenerService)

for the greenlist call path — isolated behind a small JS interface; no box-firmware architecture

impact, but a new component to record on the phone side once accepted.

Patterns incorrectly followed

None identified. The design stays within the documented pin-map, module, config, and NVM conventions,

and preserves the seven fixed product functions (it adds a second interface onto them rather than

changing them).

## 9. Confidence level

75/100 for shipping the viewer MVP on this approach. The architecture is well-trodden and every

box-side integration point already exists. The 25 points of doubt are: (a) two unverified board facts

(_bleio/sdioio in this CP build) that a one-hour on-device spike resolves, and (b) the genuinely

hard, platform-imposed background-BLE/greenlist behavior on iOS — which the MVP deliberately avoids.

## 10. Recommended immediate next steps

Board capability spike (blocking): on the real board, confirm import _bleio / adafruit_ble

and sdioio/sdcardio, then advertise a trivial PhoneBox service and connect from a generic BLE

scanner app (nRF Connect). One session; resolves the top risk.

Firmware SD-logging seam: add the go_done session-record emit + SD sink + on-device stats view.

Pick the mobile stack (RN unless the team is already Flutter-native) and stand up the BLE service

layer against the trivial service from step 1.

Build the viewer MVP, validate against the table in §7, then proceed down the roadmap.

## Sources

Board / SD slot & BLE: Waveshare ESP32-S3-Touch-LCD-1.47

CircuitPython BLE peripheral/GATT: `_bleio` docs, Adafruit `adafruit_ble` library

CircuitPython SD interfaces: `sdioio`, `sdcardio`

RN BLE library: `react-native-ble-plx` (dotintent), Expo config plugin, background mode (iOS) wiki)

Cross-platform vs native BLE: Novelbits — native vs cross-platform BLE, LogRocket — comparing RN BLE libraries

Flutter BLE: `flutter_blue_plus`

iOS background BLE: Apple — Core Bluetooth background processing, Punch Through — leveraging background Bluetooth

iOS call greenlisting: Apple — `CXCallObserver`, Apple — Identifying and blocking calls, Apple — PushKit / VoIP, VideoSDK — CallKit VoIP guide

Android notification access (greenlist): NotificationListenerService

BOM — original board: The Pi Hut ESP32-S3-Touch-LCD-1.47, Amazon B0F8B8JF74, CircuitPython board

BOM — radio-free option: Waveshare RP2350-Touch-LCD-1.28 (Pi Hut), RP2040-Touch-LCD-1.28 (Pi Hut), CircuitPython RP2350 board

BOM detail & board shortlist: docs/procurement/bom.md, docs/procurement/board-cost-reduction/02-supplier-longlist-and-shortlist-2026-07-19.md

Upstream feature context: docs/brainstorming/phone-box-feature-ideation-2026-07-20.md