# Phone Box — Development Roadmap

> DRAFT — Requires Human Approval

> Date: 2026-07-24

> Owner: light-coder / solo maintainer. Planning artifact only — no source was changed to produce this.

Where the product stands today. The Phone Box is a CircuitPython touchscreen phone-lock box running on a Waveshare ESP32-S3-Touch-LCD-1.47 (board now locked — kept for its onboard LiPo charging and its BLE radio), paired with an unbuilt React Native + Expo companion app. The firmware is surprisingly complete: SD session logging, an on-device "Focus" stats view, a full 6-characteristic BLE GATT peripheral, controller command hooks, an incoming-call overlay, and a MAX17048 fuel-gauge driver are all written and wired into the run loop. The catch is that almost none of it is *verified*, and the whole BLE feature set silently self-disables because the adafruit_ble library is not vendored into the board. The app is mostly real (not mocks) but has never been compiled or run, and its whole on-device path is blocked behind an Apple toolchain the owner does not currently have. In short: the code is far ahead of the verification. This roadmap orders work to de-risk cheaply — confirm what already exists on local hardware before spending on the app track — and runs a parallel, low-risk cost-down effort that needs no new code.

## 1. Current-State Snapshot

### Firmware (Box-code/)

| Feature | State | Notes |
| --- | --- | --- |
| Timer / servo lock / physical override / power mgmt / NVM settings / brownout-safe boot | Done (7 fixed functions) | Must be preserved by any cost cut. |
| SD session logging (lock_log.py) + on-device Focus stats | Built, unverified | Wired into run loop; self-disables to "NO SD CARD" without a writable microSD + sdioio in the CP build. |
| BLE GATT peripheral (lock_ble.py, 6 chars) + command hooks + call overlay + time-sync | Built, silently disabled | ImportError on adafruit_ble/_bleio → whole peripheral early-returns enabled=False. One missing dependency kills all four features. |
| BLE security (pairing / bonding / encryption) | Not started | All 6 characteristics open read/write. Only mitigations: remote unlock off by default + 1s rate-limit. |
| MAX17048 fuel gauge (max17048.py + lock_battery.py) | Built, unverified | External Adafruit #5580 breakout on I2C 0x36; reads 0% / "unavailable" if not physically wired. Accuracy *upgrade*, not required. |
| time_sync UI consumer | Dead end | Value is stored but never displayed. Minor. |
| Host test suite | None | All firmware verification is on-device only. |

### Companion App (app/, React Native + Expo + TS)

| Feature | State | Notes |
| --- | --- | --- |
| Protocol codec + focus-stats logic | Real + unit-tested | Runs on Windows via npm test. |
| BLE client (PhoneBoxClient.ts) — scan/connect/notify/read/write | Real, never run | Needs a real board advertising the service UUID. |
| zustand store, viewer dashboard | Real, read-only | startLock / writeSettings exist in store but no UI wires them. |
| Native iOS CallObserverModule.swift | Real, won't compile | No `.podspec` → autolinking/prebuild fails. iOS-only, no Android. |
| Tier-1 any-call "alert-through" | Done (logic) | Never run on device. |
| Tier-2 per-contact greenlist | Stub | resolveLabel returns 'Call'; CXCallObserver cannot supply caller ID by design — needs VoIP/PushKit + a real iOS dev. |
| Session-history sync | Not started (both sides) | Box exposes aggregate stats only; no chunked per-session history characteristic exists on box or app. |
| Accountability / cloud tier | Not started (greenfield) | No backend, auth, or sync anywhere. |
| Build environment | Windows-only blocker | Only npm test + npm run typecheck run locally. On-device needs macOS + Xcode + paid Apple Dev + iPhone; no eas.json configured. |

### Cost / BOM

As-built prototype ≈ $43.77 retail. No-redesign cost-down floor ≈ $28/unit.

Board locked to the Waveshare ESP32-S3-Touch-LCD-1.47.

## 2. Phased Roadmap

Ordered to de-risk: cheap, local, verifiable work first; expensive Apple-toolchain and greenfield work last. The cost-down track (Phase 5) runs in parallel and depends on nothing.

### Phase 1 — Verify SD logging + on-device Focus stats

Goal: Confirm the already-built logging + stats path actually writes and reads on real hardware.

Builds on / depends on: Existing lock_log.py; nothing upstream.

Effort & cost: ~0.5 day. ~$8–12 for a microSD card (may already be on hand).

Current blocker: Need an inserted, writable microSD card and sdioio present in the board's CircuitPython 10.2.1 build. Without both, the feature silently degrades to "NO SD CARD" and you'll never know it works. (Blocker B3.)

### Phase 2 — Bring the BLE stack online (THE gate)

Goal: Get lock_ble.py to initialize with enabled=True, unlocking BLE + command hooks + time-sync + call overlay in one move.

Builds on / depends on: Nothing upstream, but gates almost everything downstream.

Effort & cost: ~0.5–1 day. $0 BOM (radio already on board).

Current blocker: adafruit_ble/_bleio is not vendored in Box-code/lib/ and its presence in the board's CP 10.2.1 build is unconfirmed. On ImportError the whole peripheral early-returns disabled. This single blocker kills four firmware features at once — highest-leverage fix in the project. (Blocker B1.)

### Phase 3 — Verify MAX17048 battery gauge (optional accuracy upgrade)

Goal: Confirm accurate battery % reporting via the fuel gauge.

Builds on / depends on: Independent; nice-to-have, not on the critical path.

Effort & cost: ~0.5 day. +$2–6/unit — this is an *upgrade*, not a cost cut. Basic battery display (one of the 7 fixed functions) does not require it.

Current blocker: The Adafruit #5580 breakout must be physically wired to I2C 0x36; unwired it degrades to "unavailable" (reads 0%). (Blocker B6.)

### Phase 4 — Lock in the app's local verification baseline

Goal: Keep the codec / focus-stats / store logic green as a regression net before any device work.

Builds on / depends on: Existing unit tests.

Effort & cost: ~0.25 day. $0 — runs on the current Windows machine.

Current blocker: None — ready to build. npm test + npm run typecheck already run on Windows.

### Phase 5 — Execute the no-redesign cost-down (parallel track)

Goal: Drive as-built ~$43.77 toward the ~$28/unit floor without touching the 7 fixed functions.

Builds on / depends on: BOM ground truth; independent of all app/firmware verification.

Effort & cost: ~1 day sourcing + one print-profile tweak + one minor UI change. Net savings target ≈ $15/unit.

| Lever | Saving | Redesign? |
| --- | --- | --- |
| Buy same board from The Pi Hut (~$18.50 vs $24.99 Amazon) | ~$6.49 | No |
| Bulk-source estimated lines | ~$6 | No |
| Optimize the print | ~$1.20 | No |
| Move lock button on-screen (drop a switch + cutout; keep physical override) | one switch + cutout | Minor firmware/UI |
| Battery 2000mAh → 1000mAh (~$3.50) | ~$7 | Only if measured runtime allows |

Current blocker: None to start on sourcing/print levers. The 1000mAh battery swap is gated on measured runtime data (produce it during Phase 1/3 bench testing).

### Phase 6 — BLE read-only Viewer MVP (app track begins)

Goal: Ship the RFC's Step-2 MVP — app scans, connects, and displays live stats from the box.

Builds on / depends on: Phase 2 (BLE live), Phase 4 (green tests). Matches approved RFC Step 2.

Effort & cost: If hired out, ≈ $5k–20k / 5–10 weeks. DIY is cheaper but gated on tooling below.

Current blocker: Windows-only environment cannot build iOS. Needs macOS + Xcode + paid Apple Developer account + physical iPhone (B2), the missing `.podspec` fixed so the native module autolinks (B4), and a real board advertising the service UUID (B1, cleared in Phase 2). Workaround for the macOS gap: configure `eas.json` for EAS cloud-macOS builds (B9).

### Phase 7 — Add session-history sync (both sides)

Goal: Give the viewer a real per-session history dashboard, not just aggregates.

Builds on / depends on: Phase 2 + Phase 6; Phase 1 (SD is the history data source).

Effort & cost: ~1–2 weeks (firmware chunked characteristic + app consumer + tests).

Current blocker: No chunked per-session `history` characteristic exists on either side — must be designed and added to the box GATT table and the app client. (Blocker B7.)

### Phase 8 — Write path: remote lock, scheduled/recurring locks, profiles

Goal: Wire the existing startLock / writeSettings store actions to UI; add scheduling + profiles. (RFC Step 3.)

Builds on / depends on: Phase 6 (viewer), Phase 2 (command hooks live).

Effort & cost: ~2–4 weeks app + minor firmware.

Current blocker: Store actions exist but no UI wires them — and shipping a write path over an unauthenticated BLE link is unsafe, so Phase 9 must land before broad release. (Blockers B5 gating safe ship.)

### Phase 9 — BLE security: pairing / bonding / encryption

Goal: Close the open read/write surface before remote unlock ships to anyone but the owner.

Builds on / depends on: Phase 2. Gates safe shipping of Phase 8 and Phase 10.

Effort & cost: ~1–2 weeks firmware + app; depends on what CP _bleio bonding supports on this chip (verify during Phase 2).

Current blocker: No pairing/bonding/encryption at all — any BLE client can read stats and write commands/settings/alerts. Current-only mitigations (remote unlock off by default + 1s rate-limit) are not sufficient for a shipped write path. (Blocker B5.)

### Phase 10 — Accountability / family mode (+ optional cloud backend)

Goal: RFC Step 4 — multi-user accountability, possibly pulling in a cloud backend (RFC "B3").

Builds on / depends on: Phase 8 (write path), Phase 9 (security).

Effort & cost: Large. Greenfield backend, auth, sync — needs a real dev; multi-week to multi-month.

Current blocker: Not started anywhere — no backend, auth, or sync exists. (Greenfield.)

### Phase 11 — Greenlist calls (iOS-first)

Goal: RFC Step 5. Tier 1 = any-call "alert-through"; Tier 2 = per-contact greenlist; Android later.

Builds on / depends on: Phase 6 (device build working), Phase 2 (call overlay live on box).

Effort & cost: Tier 1 is mostly done in logic. Tier 2 is a research-grade spike (VoIP/PushKit) needing a real iOS dev.

Current blocker: Tier-2 per-contact is a stub (resolveLabel → 'Call'); CXCallObserver cannot supply caller ID by design — Tier 2 requires VoIP/PushKit + a real iOS developer. Also gated by the Apple toolchain (B2/B4). (Blocker B8.)

### Phase 12 — OTA firmware updates over BLE

Goal: RFC Step 6 — push firmware to the box over BLE.

Builds on / depends on: Phase 2 (BLE), Phase 9 (security — do not ship unauthenticated OTA).

Effort & cost: Large, high-risk (bricking). Last for a reason.

Current blocker: Depends on secured BLE (Phase 9); no OTA transport designed yet.

## 3. Consolidated Blocker Register

Ranked by how many downstream roadmap steps each blocker gates (most-gating first).

| ID | Blocker | Gates (steps) | How to clear it |
| --- | --- | --- | --- |
| B1 | adafruit_ble/_bleio not vendored in Box-code/lib/; unconfirmed in CP 10.2.1 build → BLE peripheral early-returns disabled | 2, 6, 7, 8, 9, 10, 11, 12 | On-board REPL: import _bleio to confirm native support; if missing/incomplete, vendor the matching adafruit_ble bundle for CP 10.x into Box-code/lib/ and re-run. Verify lock_ble.py reports enabled=True. |
| B2 | No Apple build toolchain (needs macOS + Xcode + paid Apple Dev account + physical iPhone) | 6, 7, 8, 10, 11 | Acquire macOS access (Mac, or EAS cloud-macOS via B9) + enroll in Apple Developer Program ($99/yr) + have a test iPhone. |
| B4 | Missing .podspec for CallObserverModule.swift → autolinking/prebuild won't compile the native module | 6, 11 | Author a .podspec for the Expo native module so prebuild/autolink picks it up; validate with a clean expo prebuild. |
| B3 | No verified writable microSD + sdioio in build → logging silently disabled | 1, 7 (history data source) | Insert a known-good, writable microSD; confirm sdioio in the CP build; verify a session row is written and read back. |
| B7 | No chunked per-session history characteristic on box or app | 7 | Design a chunked history GATT characteristic; implement on the box, add the consumer to PhoneBoxClient.ts + store; unit-test the codec. |
| B5 | No BLE pairing/bonding/encryption; all 6 chars open read/write | 8 (safe ship), 9, 10, 12 | Implement bonding/encryption via CP _bleio (verify support during B1); until then keep remote unlock off by default + rate-limit. |
| B9 | No eas.json configured | 6 (workaround for B2) | Add eas.json; use EAS cloud macOS builds to sidestep needing a local Mac. |
| B6 | MAX17048 #5580 breakout not wired → reads 0% | 3 | Physically wire the breakout to I2C 0x36 and verify a live % reading. (Upgrade only — not on critical path.) |
| B8 | CXCallObserver can't supply caller ID by design; Tier-2 greenlist stubbed | 11 (Tier 2 only) | Prototype VoIP/PushKit-based caller identification with a real iOS dev; Tier 1 (any-call alert-through) ships without this. |

## 4. Critical Path & Recommended Next 3 Actions

Critical path (up-market / connectivity bet):

B1 (BLE online) → Phase 2 → B2/B4/B9 (Apple toolchain) → Phase 6 (Viewer MVP) → Phase 7 (history) → Phase 8 (write path) → Phase 9 (security) → Phases 10–12.

The single highest-leverage node is B1 — it gates eight downstream steps and costs $0 in BOM. The app track's realistic wall is the Apple toolchain (B2), which is a spending/logistics decision, not a coding one.

Recommended next 3 actions (all cheap, local, and de-risking):

Clear B1 today. On the board's REPL, test import _bleio; if unsupported, vendor the CP-10.x adafruit_ble bundle into Box-code/lib/ and confirm lock_ble.py comes up enabled=True. This alone revives BLE, command hooks, time-sync, and the call overlay.

Run Phase 1 + Phase 4 on the bench. Insert a writable microSD, confirm sdioio, verify a logged session round-trips, and re-run npm test + npm run typecheck. This proves the two "built-but-unverified" pillars and produces the runtime data needed to decide the 1000mAh battery cost cut.

Kick off the parallel cost-down (Phase 5). Switch board sourcing to The Pi Hut and start bulk-sourcing quotes — ~$12+/unit of savings with zero code risk while the firmware verification proceeds.

Decision the owner must make before committing to the app track: whether to fund the Apple toolchain (Mac/EAS + $99/yr Apple Dev + test iPhone) and the .podspec fix. Until that is settled, keep all effort on the firmware and cost-down tracks, which need none of it.